MIRAGE A3-108 CONTROL EMERGENCY

6TH MARCH 1969

1.	The front seat pilot of the dual Mirage was undergoing a standardization check with the Squadron 'B' Flight Commander.

At 25,000 feet and IAS 380 knots, the pilot initiated a nose high reversal to the left. From this position, the aircraft entered a nose down rolling spiral to the left at about 250 knots. The check pilot was rather confused with the front seat pilot's apparent actions, but in his own words, decided to do nothing and criticize after he had affected recovery.

2.	Meanwhile, the front pilot noticed that the aircraft had continued to roll unabated, and as the aircraft was not obviously in a spin, he could not think of any reason for the unusual behaviour of the aircraft.

As the aircraft passed through 17,000 feet, the check pilot became alarmed and took over the controls, at the same time asking the front pilot for an explanation. The check pilot applied full right aileron and rudder to stop the roll, coupled with full forward stick to overcome a strong pitch up tendency.

3.	The aircraft was recovered at 14,000 feet and 300 knots, in an uncomfortable left wing down attitude, with all controls hard over to the right and stick almost fully forward. The check pilot then asked the front pilot to select Emergency Disengage, and to pull the pitch and yaw circuit breaker. This action resulted in a violent pitch up despite full forward stick and was countered by rolling quickly into a left turn. Emergency Disengage and damping were promptly reselected to their normal position and control of the aircraft was regained.

A short discussion on ejection drills ensued, and then the check pilot noticed that the port inner elevon was in the fully up position. This explained the aircraft's behaviour and it was decided to check controllability at lower airspeeds. At 200-250 knots the aircraft was controllable, and it was possible to turn starboard. Additionally at the lower speeds with the undercarriage down, less forward speed was required for level flight.

4.	Shortly afterwards, another Mirage joined in formation at a safe distance, and reported that the port inner elevon was the only apparent unserviceable control surface, and that the pitch dampers were deflected well down from the central position. The dampers were compensating to some degree, and this explained the pitch up when they were previously disengaged.

5.	Fuel contents were now 180 gallons, and the aircraft was 30 miles from base, at 10,000 feet. The two pilots discussed actions for ejection in event of loss of control, and the decision was made to land the aircraft.	The check pilot decided not to use 'approach control' mode, because he felt that a reduction of speed below 200 knots was not warranted under the circumstances. The aircraft was positioned for a high straight in approach and at four miles the check pilot gave control to the front pilot, whilst retaining throttle control from the rear cockpit. The front pilot had to use both hands to control the aircraft, and the final approach was made at 200 knots. The landing was successful, with the rear pilot deploying the brake chute after touchdov4n.

Comment

6.	The aircraft captain (Squadron Leader T.C. Carter) handled a dangerous situation extremely well, and the front seat pilot (Flying Officer K.I. Semmler) is to be congratulated for his skilful flying of the Mirage on final approach and landing. Both pilots have been awarded Green Endorsements in their logbooks.

The incident was caused by disconnection of the Link Rod Assembly from the Servo Control of the port inboard elevator.	This resulted in the elevon driving to maximum deflection. The disconnection occurred because the securing nuts, washers, and split pins of the elevon had not been fitted to the Link Rod Assembly at the last ‘D’ Servicing. An independent check inspection of the flight controls was not carried out, as required by ABO T 25/12, and thus the omission by the airframe fitter was undetected. It is difficult to understand how the aircraft had flown 77.00 since the ‘D’ Servicing without the elevon control failing, but it was thought probable that the self-aligning bearing, for which the bolt is a sliding fit, may have contributed towards stabilizing the bolt, thus reducing the probability of it coming off.

34.	During a tactics training exercise at 20000 ft in a nose high attitude at 300 kts the aircraft entered a left-hand rolling spiral from which recovery was affected with great difficulty at 14000 ft with full forward and full right stick and full right rudder. A MAYDAY was broadcast, and an ejection planned: however, a reduction of speed to 220 kts restored enough control to consider landing.	Visual inspection from the cockpit showed the port inner elevon jammed hard up in an abnormal position. As the aircraft handled safely at 200 kts with u/c down, a long straight in approach was affected with front seat control of stick and rudder and rear seat control of throttle to touchdown at 100 kts.

Cause:	Loss of bolt from link rod.

Court of Inquiry convened.