MIRAGE A3-8 ABORTED TAKE OFF ESL

17TH FEBRUARY 1978

Note 1:

Mirage A3-8 laden with four 500lb Mk 82 HE bombs, and Defa ammunition crashed into the arrester barrier as a result of an aborted take-off. The aircraft caught fire but was extinguished before the bombs and ammunition “cooked off”.

Note 2:

DAFS 58

Mirage A3-8 was No 3 in a flight of four Mirage aircraft participating in a fire power demonstration at Puckapunyal. The aircraft was configured with four Mk. 82 bombs on RPK 10 wing tanks, a centreline 286-gallon external tank and loaded with 180 rounds of 30mm ammunition.

During the take-off roll from RAAF Base East Sale, the pilot felt a 'thump’ on the left-hand side of the aircraft. The aircraft acceleration rapidly decayed, and the aircraft veered left approximately 10 - 15 degrees. Aircraft indicated speed was 150 kts with about 3,500 feet remaining. The pilot initiated an abort, deployed the drag chute, but retained all stores. Use of full right rudder, elevon and brake held the aircraft in the middle of the left-hand side of the runway.

The aircraft engaged the barrier at an entry speed of between 80 -90 knots. On engagement, the nosewheel collapsed, forcing the centreline fuel tank onto the ground, and causing it to rupture. The left main tyre had disintegrated, and the magnesium wheel caught on fire. After the centreline tank ruptured, the aircraft travelled an additional 60 feet, - spreading fuel over the area. The fire from the left main wheel ignited the fuel, and the area to the left and aft of the aircraft began burning furiously.

The pilot shut down the aircraft and safely performed ground emergency egress procedures. Emergency fire services extinguished the fire before any of the ordinance detonated.

The aircraft suffered, Category 5 damage in the resulting ground fire.

The cause of the accident was left main tyre deflation during the take-off roll. The tyre deflation in turn is believed to have been caused by incorrect wheel assembly during buildup.